Guide to using the program

“The block binary

multithreshold decoder

(MTD block)”

The program allows you to explore the effectiveness of block binary multithreshold decoder in binary symmetric communication channel. It performs the modeling of the transmission system, consisting of data source, selforthogonal code encoder, modulator, channel, demodulator, multithreshold decoder and receiver of data. The data source generates a data stream consisting of 0 and 1 appearing with equal probability. This data is supplied to an encoder which performs encoding them using a block code selforthogonal specified generating polynomial from a file encoder.txt. The result is a code word which is input to modulator performs a binary phase modulation. The generated signal is transmitted across a communication channel with additive white Gaussian noise. The system then demodulates the received signal, the demodulator makes a hard decision on the transmitted symbols. The result of demodulation is supplied to the input of the multithreshold decoder, which corrects errors in the received data. The result of the decoding is compared with the transmitted data in the receiver data, where they   count   the number of decoding errors.

The program requires three files:

1. blockMTD.exe – the executable program file.

2. encoder.txt file with settings used by the encoder.

3. param.txt file with parameters of the experiment and the decoder.

After starting the program reads the raw data of files with parameters and performs in accordance with the simulation, which estimated the probability of error at the output of the multithreshold decoder for a given signal-to-noise in the communication channel. Detailed statistics of the operation of the decoder is output to a file result.txt that is created in the current directory.

File settings param.txt:

The first three variables (EbN0_low, EbN0_high, EbN0_step) specify a range and a step change in the signal-to-noise bits in the communication channel:

EbN0_low=3.5; //start signal/noise bits

EbN0_high=5; //final signal/noise bits

EbN0_step=0.1; //pitch attitude signal/noise for a bit.

In this case it will evaluate the error probability at the output of the binary multithreshold decoder for each value of the ratio signal/noise in the channel in the interval from 3.5 to 5 dB in increments of 0.1 dB.

Following the volume parameter specifies the maximum number of transmitted bits during the experiment.

MinBlockErr parameter defines the number of erroneous blocks after the appearance of which at the current signal-to-noise can be used to terminate the simulation (this option allows for minimum time to provide the required accuracy of the obtained results).

Itert parameter specifies the number of iterations multithreshold decoding. This decoder is an iterative decoder of majority type. It   does several stages (iterations) of decoding for each   the received block. The demo-program   have a limitation of the number of decoding iterations must be at least 3 and not more than 10.

File settings encoder.txt:

nk=2 nr=2

i=1 v=1 2 6 82

i=2 v=1 2 13 141

i=1 v=2 2 0 134

i=2 v=2 2 9 207

The parameter nk specifies the number of information branches of the encoder (  this demo-program   have a limitation of no more than four information branch), the parameter nr defines the number of check branches of the encoder (this demo-program   have a limitation of no more than four test branches). When encoding the original information sequence is stored in nk shift registers of the same length (the length of registers is equal to 2p+1, where p is the maximum degree of generating polynomial). Further, using the operation of summation modulo 2, we calculate the nr of test bits. Thereafter, the cyclic shift information and registers again calculated nr testing symbols. This process is repeated, until the information registers will return to their original state (2p+1 shift).

Next, you will set polynomials connecting the i-th information and the v-th test branch. For each pair of information–the validation branch (e.g. i=1 v=1) sets the number of test symbols of the i-th information branch participating in the formation of each symbol of v-th verification branch (code in the example uses two checks), then identifies specific positions used checks, starting from zero (e.g. i=1  v=1  2  6  82 means that when the null terminator of the first test branches will be used the 6th and the 82nd symbols of the first information branch).

When iterating over all pairs of information–a check branch for indication of positions of the first checks fixed the first check branch and for it to move all the information of the branch, then fixed a second check branch for it through all of the information branch, etc.

i=1  v=1  2  6  82

i=2  v=1  2  13  141

i=1  v=2  2  0  134

i=2  v=2  2  9  207

In the demo program, the value of each of the checks should be less than 1000. If one of the positions of audits exceeds this value, the program will display the error message and immediately exits.

A sample diagram of the binary encoder of block encoder semiorthogonal code with one information and one of the check branches shown in the figure. It is seen that the encoder consists of shift register and adders modulo 2.

The use of this encoder in the file encoder.txt you need to set:

nk=1 nr=1

i=1 v=1 4 0 1 4 6

Scheme binary multithreshold decoder for such a code has the form shown in the figure.

Scheme multithreshold decoder consists only of shift registers, adders modulo 2 and threshold element (PE), whose task is the summation of their inputs and compare result with some threshold value with a further inverting the summed elements, and the decoded bits in excess of a threshold amount.

A sample diagram of a binary block encoder selforthogonal code with two information and two of the check branches is shown in the following figure.

The use of this encoder in the file encoder.txt you need to set:

nk=2 nr=2

i=1 v=1 2 0 1

i=2 v=1 2 0 4

i=1 v=2 2 0 6

i=2 v=2 2 0 15

In the output file result.txt displays information about the simulation parameters, the modulator, the encoder and the decoder used in the simulation and for each value of the signal-to-noise in bit shows information on the estimated error probability at the output of the decoder and the number of errors remaining after each iteration of decoding.